AIML ZG535 | Machine Learning on Edge
LogiEdge Assignment Requirements & Submission Blueprint
Cold-Chain Edge AI Platform for FreightBridge Logistics

Purpose This document converts the attached assignment into an implementation-ready requirements blueprint, calculation sheet, report structure, checklist and demo runbook. It does not fabricate measured model results; items that must come from your actual execution are clearly marked as MEASURE AFTER RUN.

| Field | Requirement |
| --- | --- |
| Instructor | Pravin Yashwant Pawar |
| Submission mode | GitHub repository + demo video + final PDF report via LMS |
| Mandatory deliverables | All three are required; missing any one caps marks at 18 |
| Final report | 2,500-3,000 words, PDF, named GROUPNO_LogiEdge_Final.pdf |
| Architecture rule | Offline-first edge inference; cloud-dependent architecture is not acceptable |

# 1. Line-by-Line Requirement Traceability

Read-first principle Every task below is traceable to the attached problem statement. Use this section as the master checklist while building the repository and final report.

| ID | Requirement | Acceptance Criteria | Where to Submit |
| --- | --- | --- | --- |
| A1 | Constraint analysis | 350-500 words in final PDF: latency, bandwidth, connectivity, privacy with quantitative evidence. | Report Section 1 |
| A2 | System architecture diagram | Truck edge node, sensors, local MQTT broker, inference pipeline, local alert log, cellular uplink, operations centre. | Report Section 1 + repo PNG |
| B1 | Constraint Triangle | Compare Pi+AI HAT+, Jetson Orin Nano, STM32H7; reference 90s SLA, 10W budget, fleet cost. | hardware/hardware_justification.md |
| B2 | Roofline analysis | AI = 45 MFLOPs / 18 MB; ridge = 16 GFLOP/s / 12 GB/s; classify bottleneck. | Report Section 1 |
| C1 | Sensor simulator | CLI --anomaly {none,temp_drift,vibration,combined}; publish all streams to local Mosquitto. | data_pipeline/simulator.py |
| C2 | Preprocessing | 5-sample moving average; 30s windows step 10s; 6 features; externally saved training_stats.npy; shifted 3 sigma experiment. | data_pipeline/preprocessing.py + Phase 2 report |
| C3 | Feature-level fusion | Extract independent sensor features then concatenate into one 6-value vector; justify versus data-level and decision-level fusion. | Report Section 2 |
| D1 | Dataset and model | Generate labelled windows for 20/15/15 minutes; train MLP 32/16 ReLU; validation accuracy must exceed 88%. | training/ |
| D2 | Docker and OTA demo | python:3.11-slim; pip layers before model copy; MODEL_PATH env var; MQTT inference topic; layer-cache demo. | inference/Dockerfile + demo video |
| D3 | 10-stage pipeline mapping | Map each edge ML pipeline stage to LogiEdge-specific component, 1-2 sentences each. | Report Section 3 |
| E1 | PSI drift monitoring | 4 bins; reference on 300 clean windows; rolling 100 scores; alert above 0.25; recovery below 0.10. | monitoring/drift_monitor.py + demo |
| E2 | Ansible playbook | Exactly 7 tasks; run twice; second run must show changed=0. | deployment/logibridge_deploy.yml + demo |
| E3 | OTA strategy | Evaluate full replacement, canary 10 trucks, shadow mode; calculate bandwidth at 280 KB/model and Rs 0.10/MB. | Report Section 3 |
| F1 | Three variants | M1 FP32, M2 full INT8 PTQ >=200 calibration samples, M3 35% structured pruning + INT8. | training/models/ |
| F2 | Benchmark five metrics | Mean latency, p95 latency, size, accuracy, energy per inference. | optimisation/benchmark.py + CSV |
| F3 | Deployment recommendation | Recommend model for 85 trucks; include SLA, hardware constraints, and Class 2 recall >95%. | Report Section 4 |

# 2. Quantitative Calculations to Use in the Report

## 2.1 Bandwidth and transmission cost

The assignment gives raw sensor rates for the bandwidth analysis: temperature at 1 Hz, vibration at 500 Hz with 3 axes, and door events as discrete events. Door events are negligible compared with vibration unless high-frequency state polling is added.

| Signal | Calculation | Per day |
| --- | --- | --- |
| Temperature | 1 sample/s x 86,400 s x (8-byte timestamp + 4-byte float) | 1,036,800 bytes = 1.04 MB/day |
| Vibration raw 3-axis | 500 samples/s x 86,400 s x (8-byte timestamp + 3 x 4-byte floats) | 864,000,000 bytes = 864 MB/day |
| Total raw stream | Temperature + vibration + negligible discrete door events | ~865.04 MB/truck/day |
| Transmission cost | 865.04 MB x Rs 0.10/MB | ~Rs 86.50/truck/day; 85 trucks ~Rs 7,352/day |

Report wording to avoid assumptions State explicitly that this is a binary-light numeric payload estimate using timestamp + float32 values. If JSON payloads are transmitted, actual bandwidth will be higher. Edge processing reduces cloud traffic to alerts and synced summaries rather than raw vibration streams.

## 2.2 Roofline / Arithmetic Intensity

| Metric | Formula | Result |
| --- | --- | --- |
| Arithmetic Intensity | 45 MFLOPs / 18 MB | 2.5 FLOP/byte |
| Ridge point | 16 GFLOP/s / 12 GB/s | 1.33 FLOP/byte |
| Bottleneck classification | AI 2.5 > ridge 1.33 | Compute-bound on Raspberry Pi 5 CPU roofline |
| Optimisation implication | Reduce operations or improve compute path | INT8 quantisation, pruning, vectorisation, batching discipline, or Hailo offload |

## 2.3 Hardware fleet cost and selection

| Option | Power vs 10W budget | Pilot cost 85 trucks | Full cost 265 trucks | Decision |
| --- | --- | --- | --- | --- |
| Raspberry Pi 5 8GB + AI HAT+ | 7.5W: within power budget | Rs 12.75 lakh | Rs 39.75 lakh | Recommended: Linux, Docker, MQTT, local logging, TFLite/Hailo path, manageable cost |
| Jetson Orin Nano Super | 15W moderate load: exceeds stated AI budget | Rs 38.25 lakh | Rs 119.25 lakh | Reject for pilot: excellent TOPS but over power and high fleet cost |
| STM32H7 custom MCU | 0.4W: excellent power | Rs 2.98 lakh | Rs 9.28 lakh | Reject for this assignment deployment: too constrained for Docker/MLOps/local broker and model lifecycle requirements |

## 2.4 OTA bandwidth cost

| Strategy | Bandwidth calculation | Cost per update cycle | Recommendation view |
| --- | --- | --- | --- |
| Full replacement | 85 x 0.28 MB = 23.8 MB | Rs 2.38 | Cheap, but pushes all trucks at once; operationally risky for safety-critical cold-chain |
| Canary, 10 trucks first | Phase 1: 10 x 0.28 = 2.8 MB; if accepted all 85 = 23.8 MB total | Rs 0.28 first phase; Rs 2.38 after full rollout | Recommended: contains risk under rural connectivity and validates model before fleet-wide update |
| Shadow mode | New model sent to all 85 while old model remains active = 23.8 MB model traffic | Rs 2.38 excluding extra telemetry | Best for high assurance, but more compute/logging during shadow and slower operational cutover |

# 3. Architecture and MQTT Design Figures

Figure 1. Offline-first system architecture to include in the final report.

Figure 2. MQTT topic tree and QoS plan.

# 4. Final Report Structure: Ready-to-Fill Blueprint

## 1 - FreightBridge Deployment Context (400-500 words)

Business context: 85 refrigerated trucks in pilot, scale to 265 vehicles, pharmaceutical cold-chain risk and prior incidents.

Four constraints: latency within 90 seconds, bandwidth economics, rural connectivity gaps of 35-90 minutes, privacy through on-device inference.

Hardware selection: recommend Raspberry Pi 5 + AI HAT+ based on 7.5W, fleet cost and deployment practicality.

Include Figure 1 and roofline result: AI 2.5 FLOP/byte, ridge 1.33 FLOP/byte, compute-bound.

## 2 - Sensor Pipeline and MQTT Design (500-600 words)

Explain simulator modes and publish streams to local Mosquitto.

Insert MQTT topic tree and QoS reasons.

Describe moving average, sliding window, six features, saved training_stats.npy.

MEASURE AFTER RUN: add table comparing accuracy with correct stats versus stats shifted by 3 sigma.

Justify feature-level fusion over raw data-level and decision-level fusion.

## 3 - Model Deployment and Pipeline Mapping (400-500 words)

Map all 10 stages to actual LogiEdge components.

Explain Dockerfile layer order and MODEL_PATH switching.

MEASURE AFTER RUN: add Docker layer-cache demo result.

Add OTA cost table and recommend canary strategy.

## 4 - Optimisation and Pareto Analysis (400-500 words)

MEASURE AFTER RUN: benchmarking table with 3 variants x 5 metrics.

Insert Pareto chart from optimisation/results/pareto_chart.png.

MEASURE AFTER RUN: include Class 2 recall for recommended variant; it must exceed 95%.

Deployment recommendation must tie back to 90s SLA, device memory/flash and accuracy.

## 5 - MLOps Monitoring and Reflection (300-400 words)

MEASURE AFTER RUN: PSI values before, after combined-anomaly injection, and after recovery.

Classify drift as covariate/operational drift based on sensor conditions.

MEASURE AFTER RUN: Ansible second run changed=0.

Add one genuine technical difficulty and one future architecture change.

# 5. Ten-Stage Edge ML Pipeline Mapping

| Stage | LogiEdge Mapping |
| --- | --- |
| 1. Data sources | Truck temperature, vibration and door-event streams generated by physical sensors or simulator. |
| 2. Data ingestion | Local Mosquitto broker receives sensor messages; network uplink is not required for local processing. |
| 3. Preprocessing | 5-sample moving average smooths temperature and vibration streams; windows are formed every 10 seconds. |
| 4. Feature engineering | Each 30-second window generates six features: temperature mean/std/rate, vibration RMS/peak/kurtosis. |
| 5. Dataset labelling | Simulator mode maps to Normal, Warning and Critical classes using assignment-provided durations. |
| 6. Model training | 2-hidden-layer MLP with 32/16 ReLU units trained and validated with 20% held-out split. |
| 7. Model conversion | FP32 model converted to TFLite, with INT8 PTQ and pruned INT8 variants. |
| 8. Packaging | Inference service container includes preprocessing, runtime stats loading, model inference and MQTT publishing. |
| 9. Edge deployment | Docker image and Ansible playbook deploy model, reference distribution and container settings to edge nodes. |
| 10. Monitoring and update | PSI drift monitoring, alert logging, canary OTA and store-and-forward sync manage fleet lifecycle. |

# 6. Repository Structure and File Acceptance Checklist

logibridge/
├── README.md
├── scenario_architecture/
│   ├── constraint_analysis.md
│   └── system_architecture.png
├── hardware/hardware_justification.md
├── data_pipeline/
│   ├── simulator.py
│   ├── preprocessing.py
│   ├── training_stats.npy
│   └── mqtt_architecture.md
├── training/
│   ├── generate_dataset.py
│   ├── train_model.py
│   ├── convert_ptq.py
│   ├── prune_quantise.py
│   └── models/
├── inference/
│   ├── Dockerfile
│   ├── inference_service.py
│   └── model.tflite
├── monitoring/
│   ├── drift_monitor.py
│   └── reference_dist.json
├── deployment/logibridge_deploy.yml
├── optimisation/
│   ├── benchmark.py
│   └── results/benchmark_results.csv, pareto_chart.png
└── reports/final_report.pdf

| Checklist Item | Pass Condition |
| --- | --- |
| Validation accuracy | Greater than 88% before proceeding to conversion |
| Critical recall | Recommended model Class 2 recall greater than 95% |
| Offline operation | Inference, alerting and local logging work with cellular disabled |
| Docker OTA layer cache | Only model layer rebuilds when model.tflite changes |
| Ansible idempotency | Second run without changes reports changed=0 |
| PSI demo | PSI crosses 0.25 within 5 minutes of combined anomaly and returns below 0.10 after recovery |
| Final submission | Exactly three items: repo URL, demo link, final PDF named GROUPNO_LogiEdge_Final.pdf |

# 7. Demo Video Runbook (15-20 Minutes)

0:00-2:00 - Show repository structure, README, assignment traceability and environment.

2:00-5:00 - Start Mosquitto and simulator in normal mode; show sensor messages and preprocessing windows.

5:00-7:00 - Run inference service locally; show topic logibridge/trucks/{truck_id}/inference.

7:00-9:00 - Switch anomaly to temp_drift/combined; show Warning/Critical alert without cloud dependency.

9:00-11:00 - Show model training/validation result and TFLite conversion artifacts.

11:00-13:00 - Docker rebuild after model-only change; point out cached layers and bandwidth saving.

13:00-15:00 - PSI drift monitor: baseline, combined anomaly crossing 0.25, recovery below 0.10.

15:00-17:00 - Run Ansible playbook twice; show second run changed=0.

17:00-20:00 - Show benchmark CSV/Pareto chart and final recommendation.

# 8. Non-Fabrication Notes

What must be replaced with your own measured results Do not submit placeholder values for validation accuracy, Class 2 recall, benchmark latency, energy, PSI plots or Ansible output. These must be generated by your own live pipeline and should match what you demonstrate in the video.

# 9. Critical Implementation Rules to Verify in Code

| File | Rule |
| --- | --- |
| simulator.py | argparse must define --anomaly with exactly none, temp_drift, vibration, combined; publish to localhost broker. |
| preprocessing.py | Load training_stats.npy at runtime; never recompute normalization from live data. |
| Dockerfile | Use python:3.11-slim; pip install layers before COPY model.tflite .; MODEL_PATH env var used by inference service. |
| drift_monitor.py | Use four bins [0,0.25), [0.25,0.50), [0.50,0.75), [0.75,1.0]; rolling 100; alert message exact prefix. |
| logibridge_deploy.yml | Exactly 7 tasks matching assignment wording; second unchanged run changed=0. |
| benchmark.py | 200 measured runs after 10 warmups; report mean, p95, size, accuracy and E=Pxt energy. |